Comprehensive PPE Hazard Assessment Guideline

Engineering and administrative controls shall be used in conjunction with PPE to ensure employee 		      safety while performing tasks. Contact EHS for any questions. Specific PPE requests, including help determining size, fit, or type, or activities not covered on this PPE assessment.

| Activity | Potential Hazard | PPE Recommended |
| --- | --- | --- |
| Air Conditioning/Refrigeration Work | Water treatment chemicals, refrigerant gasses, electrical control circuits | Chemical resistant gloves, temperature resistant gloves, safety glasses, electrical safety shoes. When using large quantities of chemicals, a face shield or goggles. |
| Arc Welding or Cutting | Electric shock; metal sparks; molten and hot metal; UV, IR and visible light; falling, dropping, rolling and sharp objects. | Insulating mats and blankets; insulated/heat and puncture/cut resistant gloves; safety shoes;safety glasses; welding shield or helmet with appropriate eye shade. |
| Asbestos Work | Airborne asbestos fibers | Respirator with HEPA filter cartridges; full body disposable coverall. |
| Bird-Clean Up | Bacteria; mold spores; flying particles. | Respirator with HEPA filter cartridges, safety glasses, face shield or goggles. Personnel should be in full body coverall |
| Carpentry Work | Flying particles; falling; falling objects; cuts/punctures; lifting/carrying. | Safety glasses; safety shoes; hard hat; cut/puncture resistant gloves |
| Cement, Concrete, Brick and Block Cutting Work | Airborne silica particles | HEPA filtered respirator; full body coverall; eye protection. |
| Central Power Plant Testing & Laboratory Work | Corrosive chemicals; splashing chemicals. | Safety glasses; chemical resistant gloves; apron. When pouring large quantities of chemicals: a face shield or goggles |
| Central Power Plant Work | Hot surfaces; contact with surfaces (head); noise. | Safety glasses, heat resistant gloves; hearing protection; head protection. |
| Custodial Work | Cleaning chemicals | Chemical resistant gloves, Safety glasses. When pouring large quantities of chemicals, a face shield or goggles. When rinsing large areas with pressurized water, an apron or rain gear, Fully covered slip resistant shoes |
| Elevator Maintenance Work | Unprotected electrical switches and circuits; falling; moving machinery, caught in, or between hazards. | Electrically insulated gloves (rated for energized voltage); fall protection; cut/puncture resistant gloves, safety glasses |
| Emergency Response | Hazardous chemicals, hazardous physical agents | Level A-D protection as prescribed by 29 CFR 1910.120 and onsite evaluations of hazards at time of incident |

| General Maintenance and Material Handling Work | Rough surfaced materials; lifting/carrying. | Safety glasses, cut/puncture resistant gloves; safety shoes. |
| --- | --- | --- |
| Grounds Keeping and Pavement Cleaning Work | Pesticides, noise, flying particles, rotating power tools, heat stress, insect and plant toxins | Hearing protection; safety shoes safety glasses; protection from pesticides according to label; gloves; long sleeved shirt and/or long legged pants, as needed. |
| Heavy Equipment and Pavement Cleaning Work | Noise; flying particles; rotating powered tools. | Hearing protection; safety shoes; safety glasses. |
| Electrical Work High Voltage Electrical | Electric shock, arc flash/ arc blast, explosion and burns; confined and enclosed spaces | Head protection; safety glasses; face shield; electrical safety gloves (rated for voltage of energized equipment); electrical safety shoes; electrical safety blankets or mats; non-synthetic work clothes; reflective clothing when working near vehicle traffic. |
| Electrical Work Low Voltage Electrical ( < 440 volts AC) | Electric shock; | Electrical safety gloves (rated for voltage of energized equipment); electrical safety shoes; electrical safety blankets or mats; non-synthetic work clothing; |
| Hospital Patient Isolation Room Work | Infectious agents | PAPR respirator with HEPA filters; disposable coverall or hospital gown; rubber gloves. |
| Laboratory Maintenance Work | Eye hazards and potential skin contact with chemical, biological or radiological residues | Safety glasses, closed toe shoes, long pants. Additional PPE such as a lab coat or coverall and light resistant gloves (latex, vinyl, or nitrile) may also be needed depending on the lab hazards. See door signage for details. |
| Laborer Work | Flying particles; falling; falling objects; cuts/punctures; lifting/carrying. | Safety glasses; safety shoes; hard hat; cut/puncture resistant gloves; fall protection when working at elevations. When using high speed powered tools, a face shield. |
| Lead Work | Lead aerosols | HEPA filtered respirator; full body coverall; eye protection. |
| Locksmithing Work | Flying particles. | Safety glasses; face shield when using high speed powered tools. |
| Masonry and plastering work | Flying particles; falling; falling objects; cuts/punctures; lifting/carrying. | Safety glasses; safety shoes; hard hat; cut/puncture resistant gloves; When using high speed powered tools, a face shield. |

| Metal Cutting and Grinding | Noise, flying particles, metal sparks and chips. | Heat and puncture/cut resistant gloves; safety glasses; face shield. |
| --- | --- | --- |
| Sheet Metal Work | Noise, flying particles, metal sparks and chips. lifting/carrying, sharp edges | Safety glasses, face shield for high speed tools, puncture/cut resistant gloves, safety shoes |
| Moving work | Lifting/carrying, dropping, rolling and sharp objects | Cut/puncture resistant gloves, Safety shoes |
| Oxy-fuel Welding or Cutting | Metal sparks; molten and hot metal; UV, IR and visible light; falling, dropping, rolling and sharp objects. | Heat and puncture/cut resistant gloves; safety shoes; safety glasses; welding shield or helmet with appropriate eye shade.Cotton Clothes or welding jacket. |
| Painting / Paint shop | Lifting/carrying; vapors, mists, solvents, chemicals and flammables | Safety shoes; organic vapor cartridge respirator w/ pre-filter, chemical resistant gloves; apron and face shield when pouring or working with large quantities of chemicals. |
| Painting work | Flying particles; falling; falling objects; cuts/punctures; lifting/carrying; coatings and solvents | Safety glasses; safety shoes; hard hat; cut/puncture resistant gloves; chemical resistant gloves when handling chemicals. When using high speed powered tools, a face shield. |
| Pest Management Work | Pesticides, animals and plants | Protection from pesticides according to label; gloves; long sleeved shirt and/or long legged pants, as needed. |
| Plumbing Work | Hot surfaces, rough surfaced materials, sewage; lifting and carrying | Safety glasses, heat resistant gloves, cut/puncture resistant gloves or latex rubber gloves; safety shoes |
| Pool Maintenance and Water Treatment Work | Water treatment chemicals; large quantities of water. | Safety glasses; chemical resistant gloves; face shield and apron or coveralls when pouring large quantities of chemicals; over boots when working in standing water; flotation device when working near or over water deep enough to be a drowning hazard. |
| Working at heights | Falls | Fall protection, Guardrails or Personal fall arrest systems per JHA |
| Refuse Work | Noise; sharp objects; garbage | Safety glasses, hearing protection; cut/puncture resistant gloves; rubber gloves; safety shoes. |
| Roofing work | Falling; Hot surfaces; heat and cold stress; sealing chemicals;solvents; lifting/carrying | Safety glasses; safety shoes; fall protection; heat resistant gloves; chemical resistant gloves; Appropriate weather related clothing. |

| Sheet Metal Work | Flying particles; lifting/carrying; rough surfaced materials | Safety glasses; safety shoes; puncture/cut resistant gloves. When using high speed powered tools, a face shield |
| --- | --- | --- |
| Silica Work | Airborne silica particles | HEPA filtered respirator; full body coverall; eye protection. |
| Striping Work | Flying particles; lifting/carrying; paints and solvents | Safety glasses; cut/puncture resistant gloves; chemical resistant gloves when handling chemicals. |
| Torch Brazing | Metal sparks; molten and hot metal; UV, IR and visible light. | Heat and puncture/cut resistant gloves; filter lens spectacles or goggles, or safety glasses and face shield, with appropriate eye shade (at least #3). |
| Torch Pavement Marking and Patching | Open flame; hot liquid material. | Heat resistant gloves; safety glasses |
| Torch Soldering | Molten and hot metal, UV, IR and visible light. | Heat and puncture/cut resistant gloves; filter lens spectacles or goggles, or safety glasses and face shield, with appropriate eye shade (at least #2). |
| Tree Trimming Work | Pesticides, falling, falling objects, noise, flying particles, heat stress, insect and plant toxins | Fall protection; hearing protection; hard hat; protection from pesticides according to label; safety shoes; safety glasses; gloves; long sleeved shirt and/or long legged pants, as needed |
| Tunnel Work | Contact with surfaces (head); heat stress; hot surfaces. | Head protection; light clothing; temperature resistant gloves. |
| Vehicle/Equipment Maintenance Work | Flying particles; petroleum solvents and wastes; falling, dropping, rolling and sharp objects | Safety glasses; chemical resistant gloves; safety shoes. |
| WoodWorking /Cabinet shop Work | Noise; flying particles; lifting/carrying; rough surfaced materials. | Hearing protection; safety glasses; face shield when using high speed powered tools; puncture/cut resistant gloves; safety shoes. |