Lock-out Tag-out Procedure                                 Rev. 1.2 11/17/2020

| Equipment Name Asset I.D.: | Location: |
| --- | --- |

F&O LOCKOUT APPLICATION

Step 1	Notify all affected employees of the planned shutdown/lockout

Step 2	Identify location of energy isolating device and the magnitude of energy. (e.g., Electrical, Mechanical, Thermal, Chemical, Hydraulic, Pneumatic, Gravitational, Stored Energy)

Step 3	Shut down equipment using the normal stopping procedure.

Step 4	Isolate equipment at the disconnects.

Step 5	Apply assigned Locks and Tags to each of the isolation points in the table below.

Step 6	Dissipate or restrain residual or stored energy (e.g., pressurized gas, rotating parts from chimney effects, suspended objects, etc.)

Step 7	Verify the isolation is effective by attempting to start the equipment and/or testing for zero energy state of exposed conductors.

| Energy Type & Source | Lockout Location | Means of Isolating and/or Dissipating Stored Energy | Verify Procedure |
| --- | --- | --- | --- |

F&O LOCKOUT REMOVAL

Step 1 	Ensure nonessential items have been removed and that the equipment is intact. (tools removed and covers replaced)

Step 2 	Ensure that all personnel have been safely positioned or removed from the work area.

Step 3 	Verify that the equipment controls are in neutral or off position.

Step 4	Each individual who applied a lockout or tagout device shall remove their own device.

Step 5  	If equipment is ready to be put back into service, notify the affected personnel that work is completed and equipment can be reenergized.

| CATEGORY | KEY WORDS |
| --- | --- |
| ENERGY SOURCES | Chemical, Electrical, Gas, Hydraulic, Mechanical, Pneumatic, Stored Energy, Thermal, Kinetic, Gravity and Other. |
| MAGNITUDE | The amount of hazardous energy in Volts, PSI, and Temperature. |
| STORED ENERGY | Block, Open/Bleed Valve, Discharge Electrical, Allow temperature to reach safe level. |
| MEANS OF ISOLATION | Throw Disconnect, Flip Breaker, Switch Off, Turn Valve Off, Dissipate, Block, Brace. |
| LOCATION OF ISOLATION | Local*, N, S, E, W, NW, NE, SW, SE of equipment and Other. |

Step 6  	If equipment is NOT ready to be put back into service, then apply out-of-service tag.  Equipment must in safe condition that would not cause immediate injury due to restoring energy; (see out-of-service tag procedure).

*LOCAL IS DEFINED AS BEING IN VISUAL PROXIMITY FROM EQUIPMENT BEING SERVICED OR MAINTAINED.

HAZARDOUS ENERGY THRESHOLDS

| Energy Form | Evaluate Hazard and Consider Lockout/Tagout | Lockout/Tagout Required (see note 1) |
| --- | --- | --- |
| Electrical (AC or DC) | < 50V and < 5mA, and ≤ 10J | ≥ 50V, or > 5 mA or > 10J |
| Thermal (hot) | Liquids or gases ≤ 125°F (52°C) Surfaces ≤ 140° F (60°C) | Liquids or gases > 125°F Surfaces ≥ 140° F |
| Thermal (cold) | Liquids and surfaces ≥ 27°F (-3ºC) | Liquids and surfaces < 27°F |
| Mechanical - kinetic | No threshold; each situation must be evaluated |  |
| Mechanical - potential | No threshold; each situation must be evaluated |  |
| Pneumatic | ≤ 150 psi in rigid pipe ≤ 50 psi in flexible, unsecured lines | > 150 psi in rigid pipe (see note 2) > 50 psi in flexible, unsecured lines |
| Hydraulic | ≤ 150 psi in rigid pipe ≤ 50 psi in flexible, unsecured lines | > 150 psi in rigid pipe (see note 2) > 50 psi in flexible, unsecured lines |
| Chemical | No threshold; each situation must be evaluated based on the chemical’s hazardous properties | Also refer to SDS (safety data sheets) |
| 1. Unless de-energizing the source by lockout/tagout introduces additional or increased hazards or is infeasible due to equipment design or operational limitations. 2. Double valve isolation is required when the operating temperature exceeds 200°F or the operating pressure exceeds 500 psig. |  |  |